1. Where is the RISC-V program located in the vsd-riscv2 respository?

The RISC-V program is stored in the samples directory of the vsd-riscv2 repository as the file sum1ton.c.

2. How is the program compiled and loaded into memory?

The program is edited using gedit, compiled with riscv64-unknown-elf-gcc to generate sum1ton.o, and executed using the Spike simulator and loaded in memory by proxy kernel (pk).

3. How does the RISC-V core access memory and memory-mapped I/O?

The RISC-V core accesses instructions and data from simulated memory created by Spike.

Memory-mapped I/O operations like printing are handled through system calls managed by the proxy kernel.

4. Where would a new FPGA IP block logically integrate in this system?

A new FPGA IP block can be added at the RTL level inside the basicRISCv/RTL directory and accessed through memory-mapped addresses.